ELEMENTOS REQUERIDO PARA CITIOS DE REPETICION

Repetidora

ESPECIFICACIONES TÉCNICAS

GTR 8000 Base Radio Specifications for Trunked Simulcast and

Conventional UHF R2 (435–524 MHz)

Table 1-12   GTR 8000 Base Radio General Specifications for Trunked Simulcast and Conventional

UHF R2 (435–524 MHz)

General Specifications

Model Number	T7039A

T7540A (where applicable)

Number of Channels	1

Size (H x W x D)	133mm x 483mm x 457mm (5.25" x 19" x

18")

Weight	21 kg (46 lbs) Temperature Range

Operating:   -30 to 60°C (-22 to 140°F)

UHF R2 (435–524 MHz) (Continued)

General Specifications

Storage:   -40 to 85°C (-40 to 185°F)

Operating Altitude	Up to 1800 meters (5900 feet) above mean sea level

Above 1800 meters (5900 feet), the derating is 1.5°C/km (0.8°F/1000 feet)

Maximum operational altitude is 5000 meters

(16900 feet)

Power Requirements	90-264 VAC, 47-63 Hz

43.2-60 VDC

Power Consumption

AC:   C4FM: 460 W max

H-DQPSK, LSM: 510 W max

DC:   C4FM: 420 W max

H-DQPSK, LSM: 470 W max

Power Supply Type	Switching Battery Revert	Included Input/Output Impedance	50 Ohms

Antenna Connector Types

Frequency Stability

Tx:   N female

Rx:   BNC female without preselector

N female with preselector

Aging:

Internal Reference (transceiver option card)

Frequency Stability

External Reference

30 ppb/yr

100 ppb/5yr

Temperature: 40 ppb

TRAK

Frequency Generation	Synthesized

Table 1-13   GTR 8000 Base Radio Transmitter Specifications for Trunked Simulcast and

Conventional UHF R2 (435–524 MHz)

Transmitter Specifications

Frequency Range	435–524 MHz

Power Output*	2-110 W C4FM

2–100 W LSM, H-DQPSK

Channel Spacing	12.5 kHz

Electronic Bandwidth	Full Bandwidth

Table 1-13   GTR 8000 Base Radio Transmitter Specifications for Trunked Simulcast and

Conventional UHF R2 (435–524 MHz) (Continued)

Transmitter Specifications

Modulation	C4FM, LSM,

H-DQPSK

Modulation Fidelity	5%

Spurious and Harmonic Emissions

Attenuation

90 dB

Emissions Designators	8K70D1W, 8K10F1E, 9K80D7W Adjacent Channel Power Ratio

12.5 kHz offset, 6 kHz BW:   67 dB

Tx Noise in Rx Band	-142 dBc/Hz

Intermodulation Attenuation	65 dB

*Full transmitter output power is available during battery revert.

The output power reference plane is the output connector of the power amplifier. The loss of the transmitter output cable (PA output to back of base radio) is 4% at 800 MHz and 3% at 450 MHz and 1% at 150 MHz. However, the base radio software allows the transmitter output power to be set at 10% above rated value.

Table 1-14   GTR 8000 Base Radio Receiver Specifications for Trunked Simulcast and

Conventional UHF R2 (435–524 MHz)

Receiver Specifications Frequency Range	435–524 MHz Static Sensitivity 5% Bit Error Rate (BER)

C4FM:   -118 dBm

H-CPM:   -116 dBm

Faded Sensitivity 5% Bit Error Rate (BER)

C4FM:   -110 dBm Intermodulation Rejection	85 dB Adjacent Channel Rejection	60 dB

Spurious and Image Response Rejection	85 dB

Spurious and Image Response Rejection with

Base Radio Preselector

100 dB

Signal Displacement Bandwidth	1 kHz

Intermediate Frequencies

1st:   73.35 MHz

Table 1-14   GTR 8000 Base Radio Receiver Specifications for Trunked Simulcast and

Conventional UHF R2 (435–524 MHz) (Continued)

Receiver Specifications

|  | 2nd: | 2.16 MHz |
| --- | --- | --- |
| Electronic Bandwidth |  | Full Bandwidth |
| Blocking Immunity |  | 100 dB |
| Conducted Spurious |  | -57 dBm |
| Bit Error Rate Floor |  | 0.01% |
| Co-Channel Rejection |  | 9 dB |

Table 1-15   GTR 8000 Base Radio FCC ID for Trunked Simulcast and Conventional UHF R2 (435–524 MHz)

FCC  ID

FCC Designation

Frequency Range	Type	Power Output	Type Acceptance

Number

435–512 MHz	Transmitter	2-110 W C4FM

2–100 W LSM, H-DQPSK

ABZ89FC4819

435–524 MHz	Receiver	N/A	ABZ89FR4820

Industry Canada

Table 1-16 shows the IC approval numbers, frequency range information, and power output range for the GTR 8000 Base Radio.

Table 1-16   GTR 8000 Base Radio Industry Canada for Trunked Simulcast and Conventional (UHF-R2 435–524

MHz)

IC Approval

Number	Frequency Range	Type	Power Output

IC Model

Number

| 109AB-T7039 | Tx 435–524 MHz, Rx 435–524 MHz | LSM, H-DQPSK | Variable 2–100 Watts | T7039 UHF-R2 |
| --- | --- | --- | --- | --- |
| 109AB-T7039 | Tx 435–524 MHz, Rx 435–524 MHz | C4FM | Variable 2–110 Watts | T7039 UHF-R2 |
| 109AB-4820 | Rx 450–470 MHz | N/A | N/A | T7540 UHF-R2 |

Duplexer Para Trabajo En Banda Rango 2